| Vehicle: |  |
| --- | --- |
| Date: |  |
| Mileage: |  |
| VIN: |  |

CHANGE

- Oil Filter

- Engine Oil

INSPECT

- Windshield Washer Fluid Level

- Windshield Wipers

- Engine Accessory Belts

- Engine Air Filter

- Intake Hose Condition

- Power Steering Fluid Level

- Transmission Fluid Level (check cold and warm)

- Spark Plug Wires

- Spark Plug Condition

- Brake Fluid Level

- Brake Disc Condition

- Brake Pad Condition

- Ball Joint and Dust Covers

- Steering Linkage and Boots

- Drive Shaft Boots

- Wheel Bearing (rotate hub and check for obvious damage)

- Tire Pressure

- Tire Wear

- Spare Tire

- Nuts, Bolts, Clips on Body

- Emergency Equipment

REPLACE

- Cabin Air Filter

LUBRICATE

- Propeller Shaft (as applicable)

RE-TORQUE

- Drive Shaft Bolts

PERFORM

- Rotate Tires

CLEAN

- Windshield

- Mirrors

- Headlights

- Rear Window

- Vacuum Interior

- Car Wash

- Fuel Filler Area

NOTES

Record part numbers and fluids used as well as the total cost and time spent for this service.