| Vehicle: |  |
| --- | --- |
| Date: |  |
| Mileage: |  |
| VIN: |  |

CHANGE

- Oil Filter

- Engine Oil

INSPECT

- Windshield Washer Fluid Level

- Windshield Wipers

- Intake Hose Condition

- Air Lines

- Ignition Coil Check (Resistance)

- Engine Compartment for Fluid Leaks

- Brake Lines and Hoses

- Parking Brake Adjustment

- Ball Joint and Dust Covers

- Anti-Roll Bar Linkages and Dust Covers

- Suspension Bushings and Condition

- Steering Linkage and Boots

- Steering Rack Bushings

- Power Steering Pump (check for smooth rotation with belt removed)

- Drive Shaft Boots

- Wheel Bearing (rotate hub and check for obvious damage)

- Pedal and Transmission Linkages

- Fuel Lines

- Exhaust System (Hangars and Hardware)

- Tire Pressure

- Tire Wear

- Spare Tire

- Nuts, Bolts, Clips on Body

- Door Lock Actuators

- Power Window Function

- Headlights

- Brake Lights

- Turn Signal Lights

- Marker Lights

- Dashboard and Interior Lights

- Emergency Equipment

REPLACE

- Cabin Air Filter

- Engine Air Filter

- Spark Plugs

- Spark Plug Wires

- Accessory Belts

- Brake Fluid

- Fuel Filter

- Timing Belt

- Water Pump

- Thermostat

- Engine Coolant

- Power Steering Fluid

- Transmission Filter

- Transmission Fluid

- Differential Oil

- Transfer Case Oil (4WD)

- Brake Rotors/Drums (If not replaced within 6 years)

- Brake Pads/Shoes (if not replaced within 3 years)

- Headlights (if not replaced within 3 years)

- Brake Lights (if not replaced within 3 years)

- Turn Signals (if not replaced within 3 years)

- Marker Bulbs (if not replaced within 3 years)

LUBRICATE

- Propeller Shaft (as applicable)

RE-TORQUE

- Drive Shaft Bolts

PERFORM

- Rotate Tires

CLEAN

- Windshield

- Mirrors

- Headlights

- Rear Window

- Vacuum Interior

- Car Wash

- Fuel Filler Area

NOTES

Record part numbers and fluids used as well as the total cost and time spent for this service.